Indicator Control ECU Simulation using Arduino Nano

Introduction

In modern vehicles, indicators are controlled electronically by an ECU (Electronic Control Unit). The purpose of this project is to simulate a simple indicator control system using an Arduino Nano.

The system uses two push buttons to represent the left and right indicator switches, and two LEDs to represent the vehicle indicator lights. The program controls blinking, hazard mode, and button behavior using a simple state-machine logic.

Objective

The main objective of this project is to design and implement a basic indicator control unit that can:

- Control left and right indicators

- Implement hazard mode

- Blink LEDs at fixed intervals

- Detect button press and hold events

- Handle button debounce

- Print system events through serial communication

Hardware Components

| Component | Quantity |
| --- | --- |
| Arduino Nano | 1 |
| Push Buttons | 2 |
| LEDs | 2 |
| 220Ω Resistors | 2 |
| Breadboard | 1 |
| Jumper Wires | Few |
| USB Cable | 1 |

Pin Connections

| Component | Arduino Pin |
| --- | --- |
| Left Button | D4 |
| Right Button | D5 |
| Left LED | D8 |
| Right LED | D10 |

Button Connection

Each button is connected between the Arduino pin and GND.

The program uses internal pull-up resistors, so no external resistor is needed.

LED Connection

Each LED is connected through a 220Ω resistor to protect it from excess current.

Arduino Pin → Resistor → LED → GND

System Functionality

The system works using a state machine with four main states.

| State | Description |
| --- | --- |
| IDLE | No indicators active |
| LEFT MODE | Left indicator blinking |
| RIGHT MODE | Right indicator blinking |
| HAZARD MODE | Both indicators blinking |

Indicator Operation

Left Indicator

- Press and hold the left button for 1 second

- The left LED starts blinking every 300 ms

- Press the button again to turn it off

Right Indicator

- Press and hold the right button for 1 second

- The right LED starts blinking every 300 ms

- Press again to turn it off

Indicator Switching

- If one indicator is active and the other button is pressed:

- The current indicator turns OFF

- The other indicator turns ON

Example:

Right indicator ON

Press the left button

→ Right OFF

→ Left ON

Hazard Mode

Hazard mode is activated when both buttons are pressed together.

When hazard mode is active:

- Both LEDs blink simultaneously

- Blink interval remains 300 ms

Pressing any button again exits hazard mode.

Scheduler

- The system uses a simple 100 ms scheduler to perform periodic tasks.

- Every 100 ms the program:

- Reads button inputs

- Updates the indicator state machine

Using a scheduler instead of delays ensures that the system remains responsive.

Button Hold Detection

To prevent accidental activation, the program requires the button to be held for 1 second before activating the indicator.

This ensures the system responds only to intentional inputs.

Debounce Handling

Mechanical buttons often generate noisy signals when pressed. This is called button bouncing.

The program includes a 50 ms debounce delay to ignore unstable signals and register only a valid button press.

LED Blinking Logic

The LEDs blink using a timer based on the `millis()` function.

Blink interval: 300 milliseconds

This creates a consistent ON/OFF blinking pattern similar to real vehicle indicators.

Serial Logging

The system prints important events to the Serial Monitor for debugging and demonstration.

Example output:

Indicator ECU Started

[STATE] LEFT_INDICATOR_ON

[STATE] RIGHT_INDICATOR_ON

[STATE] HAZARD_MODE_ON

This helps monitor system behavior during testing.

Testing Procedure

The project was tested using the following steps.

1. Left Indicator Test

Press and hold the left button.

Expected result:

Left LED blinks

2. Right Indicator Test

Press and hold the right button.

Expected result:

Right LED blinks

3. Switching Test

Turn on one indicator and press the other button.

Expected result:

Indicator switches correctly

4. Hazard Mode Test

Press both buttons together.

Expected result:

Both LEDs blink simultaneously

5. Reset Test

Reset the Arduino.

Expected result:

System returns to IDLE state

Conclusion

This project successfully simulates a basic indicator control ECU using Arduino Nano.

The system demonstrates concepts used in embedded systems such as:

- State machines

- Scheduling

- Button debounce

- Timer-based LED control

- Event logging

The implementation provides a simple but effective way to understand how vehicle indicator systems operate in real embedded environments.